SIMULATION 6

Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

1. Aim

To analyze microwave mixer conversion performance using the lab-manual RF/LO settings, estimate the IF output power from the assumed 8 dB conversion-loss model, and compare the result with the expected passive-mixer range of 6–10 dB.

2. Objectives

To develop simulation models representing the operating characteristics of selected microwave semiconductor devices.

To analyze the electrical characteristics and frequency-dependent behavior of Gunn, Schottky or PIN diode-based microwave structures.

To simulate frequency conversion in a microwave mixer and identify the generated frequency components.

To evaluate conversion characteristics and identify suitable operating conditions for microwave applications.

3. Input Parameters

| S. No. | Input parameter | Symbol / setting | Value (PDF / code) |
| --- | --- | --- | --- |
| 1 | RF frequency | fRF | 10 GHz |
| 2 | LO frequency | fLO | 9.5 GHz |
| 3 | RF input power | PRF | −10 dBm |
| 4 | LO power | PLO | +7 dBm |
| 5 | IF load | R (ZIF in manual) | 50 Ω |
| 6 | Assumed nominal conversion loss | CL | 8 dB (illustrative model) |
| 7 | Sampling frequency | Fs | 5 GHz (code setting) |
| 8 | Number of samples | N | 5000 (code setting) |

4. Code

clc;

clear;

close all;

%% 1. GUNN DIODE I-V CHARACTERISTICS

V = 0:0.01:10;

I_gunn = zeros(size(V));

% Ohmic region

r1 = V <= 3;

I_gunn(r1) = 5*V(r1);

% Negative differential resistance region

r2 = V > 3 & V <= 6;

I_gunn(r2) = 15 - 2*(V(r2)-3);

% High-field region

r3 = V > 6;

I_gunn(r3) = 9 + 1.5*(V(r3)-6);

%% 2. SCHOTTKY DIODE I-V CHARACTERISTICS

V_s = -0.3:0.005:0.6;

Is = 1e-9;

n = 1.2;

Vt = 0.02585;

I_schottky = Is*(exp(max(min(V_s/(n*Vt),20),-20))-1);

%% 3. PIN DIODE FREQUENCY RESPONSE

f_PIN = linspace(0.01,20,1000);

fc = 5;

H_PIN = 1./sqrt(1+(f_PIN/fc).^2);

H_PIN_dB = 20*log10(H_PIN);

%% 4. MICROWAVE MIXER INPUT PARAMETERS

fRF = 10e9;             % RF frequency (Hz)

fLO = 9.5e9;            % LO frequency (Hz)

PRF = -10;              % RF power (dBm)

PLO = 7;                % LO power (dBm)

R = 50;                 % IF load (ohms)

%% 5. THEORETICAL IF AND SUM FREQUENCIES

fIF = abs(fRF-fLO);

fSUM = fRF+fLO;

%% 6. BEHAVIORAL MIXER MODEL

% Assumed nominal conversion loss for illustration

CL = 8;

PIF = PRF-CL;

% Convert IF power from dBm to watts

PIF_W = 10^((PIF-30)/10);

% IF sinusoidal voltage across 50-ohm load

Vpeak = sqrt(2*PIF_W*R);

%% 7. GENERATE IF SIGNAL AND SPECTRUM

Fs = 5e9;

N = 5000;

t = (0:N-1)/Fs;

vIF = Vpeak*cos(2*pi*fIF*t);

Y = fft(vIF);

A = abs(Y/N);

A = A(1:N/2+1);

A(2:end-1) = 2*A(2:end-1);

f = (0:N/2)*Fs/N;

% Convert sinusoidal spectral amplitude to power

Pspectrum = (A/sqrt(2)).^2/R;

PdBm = 10*log10(Pspectrum/1e-3 + eps);

% Locate simulated IF peak

[~,idx] = max(A);

fIFsim = f(idx);

% Calculate IF power from simulated voltage

PIFsim_W = (A(idx)/sqrt(2))^2/R;

PIFsim = 10*log10(PIFsim_W/1e-3);

% Conversion loss from RF input and simulated IF output

CLsim = PRF-PIFsim;

% IF frequency deviation

devIF = abs(fIFsim-fIF)/fIF*100;

%% 8. CONVERSION PERFORMANCE VERSUS LO POWER

LOsweep = 0:1:13;

% Illustrative model; not measured device data

CLsweep = interp1([0 3 7 10 13], ...

[12 10 8 7.5 7.5], LOsweep);

%% 9. CONVERSION PERFORMANCE VERSUS RF POWER

RFsweep = -20:2:0;

PIFsweep = RFsweep-CL;

%% 10. DISPLAY NUMERICAL RESULTS

fprintf('\nMICROWAVE MIXER CONVERSION LOSS ANALYSIS\n');

fprintf('----------------------------------------\n');

fprintf('RF Frequency          = %.2f GHz\n',fRF/1e9);

fprintf('LO Frequency          = %.2f GHz\n',fLO/1e9);

fprintf('Theoretical IF        = %.3f GHz\n',fIF/1e9);

fprintf('Sum Frequency         = %.2f GHz\n',fSUM/1e9);

fprintf('RF Input Power        = %.2f dBm\n',PRF);

fprintf('LO Input Power        = %+.2f dBm\n',PLO);

fprintf('IF Load               = %.0f ohms\n',R);

fprintf('Assumed Conversion Loss = %.2f dB\n',CL);

fprintf('Expected IF Power     = %.2f dBm\n',PIF);

fprintf('Simulated IF Frequency= %.3f GHz\n',fIFsim/1e9);

fprintf('Simulated IF Power    = %.2f dBm\n',PIFsim);

fprintf('Calculated Loss       = %.2f dB\n',CLsim);

fprintf('Frequency Deviation   = %.3f %%\n',devIF);

if CLsim >= 6 && CLsim <= 10

fprintf('Result: Within expected 6-10 dB range.\n');

else

fprintf('Result: Outside expected 6-10 dB range.\n');

end

%% 11. ALL OUTPUTS IN ONE FIGURE

figure('Color','w','Position',[30 30 1500 1000]);

tiledlayout(3,3,'TileSpacing','compact','Padding','compact');

% Plot 1: Gunn diode I-V characteristics

nexttile;

plot(V,I_gunn,'b','LineWidth',2);

grid on;

xlabel('Voltage (V)');

ylabel('Current (mA)');

title('Gunn Diode I-V Characteristics');

xlim([0 10]);

ylim([0 17]);

% Plot 2: Schottky diode I-V characteristics

nexttile;

semilogy(V_s,max(abs(I_schottky),1e-12), ...

'r','LineWidth',1.8);

grid on;

xlabel('Voltage (V)');

ylabel('|Current| (A)');

title('Schottky Diode I-V Characteristics');

% Plot 3: PIN diode frequency response

nexttile;

plot(f_PIN,H_PIN_dB,'m','LineWidth',1.8);

hold on;

xline(fc,'k--','Cutoff frequency');

grid on;

xlabel('Frequency (GHz)');

ylabel('Response (dB)');

title('PIN Diode Frequency Response');

xlim([0 20]);

% Plot 4: Mixer IF output spectrum

nexttile;

plot(f/1e9,PdBm,'b','LineWidth',1.4);

hold on;

plot(fIFsim/1e9,PIFsim,'ro','MarkerFaceColor','r');

xline(fIF/1e9,'k--','Theoretical IF');

grid on;

xlabel('Frequency (GHz)');

ylabel('Power (dBm)');

title('Mixer IF Output Spectrum');

xlim([0 1.5]);

% Plot 5: Conversion loss

nexttile;

bar(1,CLsim);

hold on;

yline(6,'g--','6 dB');

yline(10,'r--','10 dB');

xlim([0 2]);

xticks(1);

xticklabels({'Simulated'});

ylabel('Conversion Loss (dB)');

title(sprintf('Conversion Loss = %.2f dB',CLsim));

grid on;

% Plot 6: Effect of LO power

nexttile;

plot(LOsweep,CLsweep,'m-o','LineWidth',1.5);

hold on;

plot(PLO,CL,'ks','MarkerFaceColor','y','MarkerSize',8);

xlabel('LO Power (dBm)');

ylabel('Conversion Loss (dB)');

title('Effect of LO Power');

legend('Illustrative model','Operating point','Location','best');

grid on;

% Plot 7: Effect of RF input power

nexttile;

plot(RFsweep,PIFsweep,'b-o','LineWidth',1.5);

hold on;

plot(PRF,PIF,'rs','MarkerFaceColor','r','MarkerSize',8);

xlabel('RF Input Power (dBm)');

ylabel('IF Output Power (dBm)');

title('Effect of RF Input Power');

legend('Illustrative model','Operating point','Location','best');

grid on;

% Plot 8: RF and LO frequency markers

nexttile;

stem([fIF fLO fRF fSUM]/1e9, ...

[PIF PLO PRF PRF-15],'filled','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Power (dBm)');

title('Mixer Frequency Components');

xticks([fIF fLO fRF fSUM]/1e9);

xticklabels({'IF','LO','RF','Sum'});

% Plot 9: IF output power versus RF power

nexttile;

plot(RFsweep,PIFsweep,'b','LineWidth',1.8);

hold on;

yline(PIF,'r--','Nominal IF output');

plot(PRF,PIF,'ko','MarkerFaceColor','y');

grid on;

xlabel('RF Input Power (dBm)');

ylabel('IF Output Power (dBm)');

title('RF-to-IF Power Relationship');

sgtitle('Microwave Semiconductor Devices and Mixer Analysis', ...

'FontSize',15,'FontWeight','bold');

%% 12. FINAL NOTES

fprintf('\nSimulation completed.\n');

fprintf('All eight analysis plots are displayed in one figure.\n');

fprintf('Device and power-sweep curves are illustrative models.\n');

5. Calculated Results

| Output parameter | Theoretical / expected value (manual) | Value from supplied code | % deviation / status |
| --- | --- | --- | --- |
| Difference frequency (IF) | ¦10 − 9.5¦ = 0.500 GHz | 0.500 GHz | 0.000% (≤ 2% limit) |
| Sum frequency | 10 + 9.5 = 19.500 GHz | Not simulated by this code | Cannot calculate |
| IF output power | Record IF power at 0.5 GHz | −18.00 dBm (model estimate) | — |
| Conversion loss | 6–10 dB expected for passive mixer | 8.00 dB (assumed-model result) | Within expected range |
| Dominant conversion product | 0.5 GHz IF; other spurs ≤ −20 dBc | 0.5 GHz IF signal generated; spurs not modeled | Partial check only |
| LO-power sweep | Interpret effect of LO power | Illustrative curve, 0–13 dBm | Not measured device data |
| RF-power sweep | Interpret effect of RF input power | Illustrative curve, −20 to 0 dBm | Not measured device data |

6. Output Parameters

| Output parameter | Expected / theoretical value | Value recorded from supplied code |
| --- | --- | --- |
| IF frequency | ¦fRF − fLO¦ = 0.5 GHz | 0.500 GHz |
| Sum frequency | fSUM = fRF + fLO = 19.5 GHz | Not generated by this code |
| IF output power | PIF = PRF − CL = −18 dBm | −18.00 dBm (behavioral model) |
| Conversion loss | CL = PRF − PIF; expected 6–10 dB | 8.00 dB; within expected range |
| IF frequency deviation | ¦fIFsim − fIF¦ / fIF × 100% | 0.000% |
| LO-power sweep | Illustrative conversion-loss curve | 0–13 dBm; illustrative only |
| RF-power sweep | Illustrative IF output-power curve | −20 to 0 dBm; illustrative only |
| Spurious components | Other spurs should be ≤ −20 dBc | Not modeled by supplied code |